Watch Method

Equipment:

- Watch (preferrably with hands), and thin twig/pencil

- One 6-8 inch straight stick or dowel

- 5-6 feet of string/twain/parachute cord

- 2- or 3-foot straight stick/dowel, and two (4-inch) pegs

Watch Method

- Hold your watch level.

- Place a short, straight twig upright against the edge of the watch at the point of the hour hand.

(Note: If using a digital watch, not the hour, imagine where the hour hand would be point, and place the twig accordingly.)

- Turn the watch until the shadow of the twig falls along the hour hand's position—that is, until the hour hand points toward the sun.

- Notice the angle formed between the numeral 12 (the top half of the digital watch) and the shadow lying on the real or imaginary hour hand.

- A line from the center of the watch that divides that angle in half will point South.

*Note:  This method requires standard time.  If your watch is set on daylight savings time, turn it back one hour.

Shadow-Stick Method

- Push a short (6-8 inch), straight stick into the ground.

- Angle it toward the sun so that the stick makes no shadow.

- Then wait until it casts a shadow at least 6 inches long.

- The shadow will be pointing east from the stick.

- A line at right angles across the shadow will be north—south.

Shadow-Stick Method

Object:   Teach how to use the sun and common objects to find directions—north, south, east, and west.

Equal-Length Shadow Method

Extra Credit:

Find directions at night using the Shadow-Stick Method and the moon

Explanation—

- The moon rises in the east and sets  in the west, as does the sun.

- The shadow-stick method for use with the sun will work just a well on the nights when the moon is bright enough to cause shadows to form.

Equal-Length Shadow Method

In the Morning—

- Push a straight 3-foot-long stick upright into the ground.

- Tie a string around the base of the stick with a bowline.

- Next, extend the string to the end of the stick's shadow.

- Tie a peg to the string at that point and use it to scratch a circle on the ground around the stick.

- Push the peg into the ground where the tip of the stick's shadow touches the circle.

In the Afternoon—

- Place another peg where the tip of the shadow again touches the circle.

- A straight line drawn between the pegs is a west-east line, with West pointing to toward the morning peg.

- A line drawn at right angles across the west-east line will be north--south.